EMU10K1 patch binary file format (.emu10k1)

-------------------------------------------

Notice: this format is a bit different from the original as10k1 assembler.

16 bytes signature 'EMU10K1 FX8010 1'	; last number means the patch version

32 bytes name				; ASCIIZ patch name

1  byte count of input/output GPRS

?  bytes input/output GPRS

1  byte count of dynamic GPRS

?  bytes dynamic GPRS

1  byte count of static GPRS

?  pairs

1 byte static GPRS

1 dword (little-endian, 32-bit) value

1  byte count of control GPRS

?  entries

1 byte control GPRS

1 dword (little-endian, 32-bit) value

1 dword (little-endian, 32-bit) range - min

1 dword (little-endian, 32-bit) range - max

32 bytes symbol name			; ASCIIZ symbol name

1  byte count of constant GPRS

?  pairs

1 byte constant GPRS

1 dword (little-endian, 32-bit) value

1  byte count of TRAM lookup tables

?  entries

1 dword (little-endian, 32-bit) size

1 byte count of read lines

? pairs

1 char address of the TRAM line

1 dword (little-endian, 32-bit) size in samples

1 byte count of write lines

? pairs

1 char address of the TRAM line

1 dword (little-endian, 32-bit) size in samples

1  byte count of TRAM delay tables

?  entries

1 dword (little-endian, 32-bit) size

1 byte count of read lines

? pairs

1 char address of the TRAM line

1 dword (little-endian, 32-bit) size in samples

1 byte count of write lines

? pairs

1 char address of the TRAM line

1 dword (little-endian, 32-bit) size in samples

2  word (little-endian, 16-bit), count of 64-bit instructions

?  double-dwords (little-endian, 64-bit) instruction